Lab 1

RC Filter and MOSFET Characteristics

(NGSpice, XSchem & ADT)

Part 1: RC Filter

τ=R*C=1ns             C=1ns1KΩ=1pF

Figure 1 RC Filter Schematic Testbench

- Transient Analysis:

Figure 2 Output Setup

Figure 3 Two Periods of the Filter

Figure 4 Rise and Fall Time simulation Results

Analytic Calculation for Rise and Fall Time:

Charging: V(t)=V0 1 - e-tRC                   V0=1V

Discharging: Vt=V0.e-tRC                            V0=1V

At Vt1=0.1⋅V0⇒t1=-RC⋅ln0.9        At Vt2=0.9⋅V0⇒t2=-RC⋅ln0.1

trise=t2-t1≈2.2⋅RC         trise≈tfall≈2.2⋅RC=2.2ns

  - Rise and Fall Time Comparison:

| Test | Output | Simulation | Analytic |
| --- | --- | --- | --- |
| ITI_Analog:Lab1_RC:1 | Fall Time | 2.196E-09 | 2.2e-09 |
| ITI_Analog:Lab1_RC:1 | Rise Time | 2.1955E-09 | 2.2e-09 |

1.2 Parametric Sweep:

Figure 5 R Parametric Sweep

Comment: Increasing the R increases the time constant thus increasing the rise and fall time, making the square wave less pronounced as R increases and the rise and fall times increase.

- AC Analysis:

Figure 6 AC Simulation Code

Figure 7 RC Filter Bode Plot (Mag)

Figure 8 RC Filter Bode Plot (Phase in Degree)

Figure 9 Max gain and BW from Simulation

Analytic Calculation for DC gain and BW:

DC Gain = 1  Cap is O.C in DC, VIN = VOUT

Bandwidth: ωp=1RC              fp=12π×RC≈159.155MHzType equation here.

| Test | Output | Simulation | Analytic |
| --- | --- | --- | --- |
| ITI_Analog:Lab1_RC:1 | Bandwidth | 159.255MHz | 159.155MHz |
| ITI_Analog:Lab1_RC:1 | Gain | 1 | 1 |

Parametric Sweep for R = 1, 10, 100, 1000KΩ

Figure 10 Parametric AC Sweep Code

Figure 11 Parametric AC Sweep Bode Plot (Mag)

Figure 12 Parametric AC Sweep Bode Plot (Phae in Degrees)

Comment: Increasing The Resistance gradually increases the Time Constant this lowering the 3dB cut off frequency decreasing the bandwidth and causing a noticeable shift for both Bode Plot graphs.

DC gain is not affected though as it does not depend on the Time Constant rather on the circuit itself.

Part 2: MOSFET Characteristics

Figure 13 MOSFET Characteristics Schematic Testbench on XSchem

Using VDD = 3.3v, Short-Channel: W/L = 10*2802/280n,  Long-Channel: W/L =  10*2.8u/2.8u

The schematic was set this way to allow for sweeping VGS and get correct results for NMOS and PMOS at the same time

Figure 14 DC Sim Code in XSchem

- ID vs VGS

Figure 15 NMOS ID vs VGS, Green: Long-Channel, Red: Short-Channel

Figure 16 PMOS ID vs VGS, Yellow: Long-Channel, Red: Short-Channel

Comment on the differences between short channel and long channel results.

• Which one has higher current? Why?

The Long Channel Device has higher current, due to the lower electric field in the channel allowing carriers to move more freely, While in short-channel the carriers’ velocity saturates limiting the current.

The effect is more noticeable in NMOS devices due to the higher carrier mobility.

• Is the relation linear or quadratic? Why?

Long-Channel: Has a quadratic relation as per the square-law Id = K*Vov2/2

Short-Channel: Has a linear relation as per the velocity saturation law.

Comment on the differences between NMOS and PMOS.

• Which one has higher current? Why?

NMOS, due to the higher mobility of electrons (Majority Carrier).

• What is the ratio between NMOS and PMOS currents at VGS = VDD?

Figure 17 Values of ID at VDD for both NMOS and PMOS

Long-Channel Current Ratio: ratio=0.0032227.04e-04=4.576

Short-Channel Current Ratio: ratio=0.0014197e-04=2.027

• Which one is more affected by short channel effects?

NMOS is more affected, due to the electrons (NMOS Majority Charge Carrier) having a higher mobility than Holes (PMOS Majority Charge Carrier).

- gm vs VGS

Figure 18  NMOS gm vs VGS, Green: Long-Channel, Red: Short-Channel (XSchem)

Figure 19 NMOS gm vs VGS, Orange: Long-Channel, Blue: Short-Channel (ADT)

Comment on the differences between short channel and long channel results.

• Does 𝑔𝑚 increase linearly? Why?

As the device turns on when VGS = Vth, gm starts increasing approximately linearly in both short and long channel as gm = k * (Vgs – Vov), but in short channel the linear increase stops early.

• Does 𝑔𝑚 saturate? Why?

It Does saturate, as Vov increases past VDSAT, gm starts to saturate, its value can be evaluated via gm= ∂I∂Vgs

- IDS vs VDS

Sweeping over: VDS = 0:VDD, and VGS = 1:1:3

Figure 20 NMOS IDS vs VDS – Long and Short Channels

Comment on the differences between short channel and long channel results.

• Which one has higher current? Why?

Long-Channel has noticeably higher current, This is due to it operating in Pinch-Off saturation which has a quadratic relation for the current as opposed to the Short-Channel NMOS which operate in Velocity Saturation which has a linear relation for the current.

• Which one has higher slope in the saturation region? Why?

Short-Channel has a higher slope in the saturation region, Due to the effect of ro.

As the length of the channel decreases the resistance between the drain and source decreases and becomes more noticeable in the short channel device.

But in the long channel device this resistance is very large, approximately infinite. resulting in no slope appearing in the saturation region.