Lab 5

Current Mirrors

Part 1: Sizing Chart

Required Spec:

| Parameter |
| --- |
| Current direction (source/sink) | Sink |
| Input Current | 10𝜇𝐴 |
| Output Current | 20𝜇𝐴 |
| % Change in Current for 𝚫𝑽𝒐𝒖𝒕 = 𝟏𝑽 | < 10% |
| Percent mismatch: 𝝈(𝑰𝒐𝒖𝒕)/𝑰𝒐𝒖𝒕 | ≤ 2% |
| Compliance voltage | ≤ 150𝑚𝑉 |
| Area | Minimize |

2) Sinking Current Means?

NMOS Transistor

3) Required Lambda

ID=K2Vov21+λVDS→ ΔID=λΔVDS→λ=0.1

5) Mismatch from ADT

Figure 1 VGMIS vs VGS

From the graph we can find the approximate value of σVT≈5.4m

6) Mismatch from PDK

varvth=0.7071*parvth*10-6WL≈5.0543m

The value from ADT is slightly higher, this is possibly due to inaccuracies that occur when calculating and estimating random values.

7) Design Parameters vs vstar at different mismatch

9) Design Parameters at required Vstar

From the Graphs we notice both values of the mismatch current satisfy the Lambda Constraint λ<0.1

Thus we will choose the dimensions corresponding with the mismatch value which has the lower area Mismatch=2%

L=1.416μm   W=14.58μm

9) What if Lambda constraints is not met?

By inputting the required exact value for lambda (λ=0.1) we can calculate the mismatch corresponding to it using the same expression used before.

Required Mismatch≈6.848%

11) Report Device Sizing and Mismatch percentage

Selected design points @ V*=150mV

| σ(Iout)/Iout | 2% |
| --- | --- |
| L | 1.42um |
| W | 14.58um |

Part 2: Current Mirror Simulation

Figure 2 Current Mirror Schematic Testbench

Design and OP Analysis

- Finding RB analytically

RB=VGS4+VDS2-VGS2IB≈VDS2IB=20KΩ

- Finding RB through Simulation

Figure 3 Value of RB from simulation

RB=27.258KΩ

The value of RB calculated from the simulation is larger than the one calculated analytically, this is due to the analytic solution not accounting for body effect which makes the difference in VGS have a value thus increasing the required RB.

- OP Analysis

Op Analysis was done using the value of RB from the simulation

|  | M0 | M1 | M2 | M3 | M4 | M5 |
| --- | --- | --- | --- | --- | --- | --- |
| gds | 3.28E-07 | 6.30E-07 | 1.31E-06 | 2.60E-06 | 3.57E-07 | 6.57E-07 |
| gm | 1.34E-04 | 2.70ES-04 | 1.33E-04 | 2.67E-04 | 1.35E-04 | 2.69E-04 |
| gmbs | 5.24E-05 | 1.05E-04 | 5.20E-05 | 1.04E-04 | 4.76E-05 | 9.54E-05 |
| ID | 1.00E-05 | 2.01E-05 | 1.00E-05 | 2.00E-05 | 1.00E-05 | 2.00E-05 |
| VDS | 7.57E-01 | 9.00E-01 | 2.00E-01 | 2.00E-01 | 5.59E-01 | 7.00E-01 |
| VDSAT | 1.20E-01 | 1.20E-01 | 1.21E-01 | 1.21E-01 | 1.22E-01 | 1.22E-01 |
| VGS | 7.57E-01 | 7.57E-01 | 7.59E-01 | 7.59E-01 | 8.32E-01 | 8.32E-01 |
| VTH | 6.78E-01 | 6.78E-01 | 6.78E-01 | 6.78E-01 | 7.53E-01 | 7.53E-01 |

- Do All Transistor operate in Saturation?

They all operate in saturation.

DC Sweep

Figure 4 Iout for both CMs vs VOUT

The cascode IOUT is more consistent and always equal to IB*2 almost during the entire sweep, while an evident error is visible in IOUT1 from the simple CM as VDS increases.

Estimate V Compliance

Figure 5 VCOMP of Simple CM

Vcomp1≈150mV

Figure 6 V compliance of Cascode Current Mirror

Vcomp2≈250mV

Figure 7 Value at which Iout1 equals IB*2 Exactly

Iout1 equals IB*2 exactly at around 0.75758mV as this is the value at which VDS of both transistor are equal each other eliminating the error

2. Simple current Mirror Error

Figure 8 Error Calculated in Simulation

Iout%Error=3.16%

The value is consistent with the results from Part one as it corresponds with the value calculated from ADT for Lambda λ=31.94m

3. Percentage of Error for both CMs

Figure 9 Current Error for both configurations

The error in the simple CM is fairly larger and fluctuates as VDS changes, it becomes zero at VDS0=VDS1 the same value calculated previously at which IOUT1=IB*2 exactly. While the cascode error is consistently zero throughout the sweep.

- ROUT vs VOUT

Figure 10 ROUT1 and ROUT2 vs VOUT

Cascode Current Mirror has significantly higher ROUT.

Yes Rout changing slightly with VOUT due to the change of operation regions of the transistors at the beginning of the sweep it has a much smaller value due to transistors operating in triode instead of saturation and even in saturation ROUT changes due to its dependance on VDS.

- Analytic Calculation for ROUT

ROUT1=ro=1gds=1.586MΩ

Rout2=gm5+gmbs5ro5ro3=213.323MΩ

Figure 11 Simulator Results

|  | Analytic | Simulation |
| --- | --- | --- |
| ROUT1 | 1.586M | 1.586M |
| ROUT2 | 213.3M | 215.43M |

Analytic and simulation results agree with each other!

Mismatch:

Mismatch on VGMIS1:

Figure 12 Error due to VGMIS1 in Simple CM

Figure 13 Error due to VGMIS1 in Cascode CM

Simple Current Mirror: IMIS1=1.269%

Cascode Current MIrror:IMIS1=1.252%

Hand Analysis

Simple Current Mirror:%IMIS1=Vmis*gm12*IB =1.2717%

Cascode Current Mirror:%IMIS1=Vmis*gm32*IB =1.25757%

Simulation and analytical results agree with each other!

Mismatch on VGMIS2:

Figure 14 Mismatch due to VGMIS2 in Cascode CM

Cascode Current MIrror:IMIS2=8.89*10-3%≈0%

Hand Analysis

Cascode Current Mirror:%IMIS2=gm5*Vmis21+gm5gds32*IB=1.2128*10-4%≈0%

5) Which mismatch contribution is more pronounced? Why?

Mismatch due to VGMIS1 is significantly more pronounced in comparison with VGMIS2, This is because it is directly affecting the mirroring devices

While mismatching in VGMIS2 only affects the cascode devices which aren’t directly responsible for the mirroring thus a way smaller errors in current is observed

6) Which design decision is better: setting the same W and L for the mirror and cascode devices? Or using larger W and L for the current mirror devices? Why

Using Larger W and L for the current mirror devices.

Because it improves mirroring as the devices are less affected by Channel Length Modulation and Process Variations and also improves the Output Resistance.

Monte Carlo Simulation: