Lab 5

Current Mirrors

Part 1: Sizing Chart

Required Spec:

| Parameter |
| --- |
| Current direction (source/sink) | Sink |
| Input Current | 10𝜇𝐴 |
| Output Current | 20𝜇𝐴 |
| % Change in Current for 𝚫𝑽𝒐𝒖𝒕 = 𝟏𝑽 | < 10% |
| Percent mismatch: 𝝈(𝑰𝒐𝒖𝒕)/𝑰𝒐𝒖𝒕 | ≤ 2% |
| Compliance voltage | ≤ 150𝑚𝑉 |
| Area | Minimize |

2) Sinking Current Means?

NMOS Transistor

3) Required Lambda

ID=K2Vov21+λVDS→ ΔID=λΔVDS→λ=0.1

5) Mismatch from ADT

Figure 1 VGMIS vs VGS

From the graph we can find the approximate value of σVT≈5.4m

6) Mismatch from PDK

varvth=0.7071*parvth*10-6WL≈5.0543m

The value from ADT is slightly higher, this is possibly due to inaccuracies that occur when calculating and estimating random values.

7) Design Parameters vs vstar at different mismatch

9) Design Parameters at required Vstar

From the Graphs we notice both values of the mismatch current satisfy the Lambda Constraint λ<0.1

Thus we will choose the dimensions corresponding with the mismatch value which has the lower area Mismatch=2%

L=1.416μm   W=14.58μm

9) What if Lambda constraints is not met?

By inputting the required exact value for lambda (λ=0.1) we can calculate the mismatch corresponding to it using the same expression used before.

Required Mismatch≈6.848%

11) Report Device Sizing and Mismatch percentage

Selected design points @ V*=150mV

| σ(Iout)/Iout | 2% |
| --- | --- |
| L | 1.42um |
| W | 14.58um |

Part 2: Current Mirror Simulation

Figure 2 Current Mirror Schematic Testbench

Design and OP Analysis

- Finding RB analytically

RB=VGS4+VDS2-VGS2IB≈VDS2IB=20KΩ

- Finding RB through Simulation

Figure 3 Value of RB from simulation

RB=27.258KΩ

The value of RB calculated from the simulation is larger than the one calculated analytically, this is due to the analytic solution not accounting for body effect which makes the difference in VGS have a value thus increasing the required RB.