Lab 7

OTA Design

Part 1: gm/ID Design Charts

NMOS Charts:

Figure 1 gm/gds vs gm/id

Figure 2 ID/W vs gm/ID

Figure 3 gm/Cgg vs gm/ID

Figure 4 VGS vs gm/ID

PMOS Charts:

Figure 5 gm/gds vs gm/ID

Figure 6 ID/W vs gm/ID

Figure 7 gm/Cgg vs gm/ID

Figure 8 VGS vs gm/ID

Part 2: OTA Design

Required Specifications:

| Technology | 0.18um CMOS |
| --- | --- |
| Supply voltage | 1.8V |
| Load | 5pF |
| Open loop DC voltage gain | >= 34dB |
| CMRR @ DC1 | >= 74dB |
| Phase margin | >= 70o |
| CM input range – low | <= 1V |
| CM input range – high | >= 1.5V |
| GBW | >= 10MHz |

Design Observations:

- The Gain required isn’t too large and easily achievable using a single stage OTA

- The CMIR required is close to the Supply Rail, Thus an NMOS input stage is preferred.

- The specification can be met using a 5T-OTA topology

We have to size 3 pairs of MOSFETS.

Figure 9 5T OTA Topology

Input Transistors:

GBW≈GMCL=gm1,22πCL=10MHz→gm1,2=314.16μs≈320μs

AOL≥34dB→50.11≈gm1,2ro2Assuming Both NMOS and PMOS have the same gds

→ro=312.5KΩ

AssumegmID=15 In Monderate Conversion→ID=21.3μA≈20μA

Assuming the Voltage Drop is divided equally across all 3 transistors in branch, VDS of each transistor = 0.6V

M1,2 Experience Body Effect, thus VSB = 0.6

We may assume a lesser voltage drop at the bottom transistor to 0.3 thus Vds = 0.9V

Using these three parameters we can get the width and length of the input pair quickly using ADT:

Figure 10 Intial Values for the input pair from SA

L=430nm     ,   W=16.72μm

These are acceptable values for the Input Pair and decent for initial design!

PMOS Current Mirror Load:

Using CMIR-Max:

CMIRMAX=1.5=VDD+Vth1,2-Vgs3,4 Using Value of Vth,2 from SA

→ Vgs3,4=1.09≈1.1V

Using assumptions from the previous part:

ro=312.5KΩ    ,    VDS=0.6V    ,    ID=20μA

Plugging in these Values in ADT SA we get:

Figure 11 Initial Values for the PMOS Current Mirror Load Pair from SA

L=320nm     ,   W=2.57μm

These are acceptable values for the Input Pair and decent for initial design!

Tail Current Mirror Sizing:

For the purposes of this design, I will use a simple current mirror at the tail.

Using the spec for CMRR:

CMRR=74dB=Ad-ACM→ACM=-40dB

→ACM=0.01≈12gm3,4ro5→1ro5=gds5≈2μs (Using Values of gm3,4 from SA)

Using the Spec for CMIR-Low:

CMIRLow=1V=Vgs1,2+V5*→V5*=100mV Extremely Low

ID=40μs→From previous assumption, twice the current flows in the tail source

Plugging in these Values into ADT SA:

Figure 12 Initial Values for the Tail Current Mirror Load Pair from SA

L=2.43μm     ,   W=674.7μm

The Value for the width is extremely high especially considering I need to multiply it by 4 for the mirroring ratio.

Thus I will assume an initial Value of W of 40um and slightly increase the Width of M1,2 then iterate the design till I reach acceptable Values for the transistor that satisfy the specifications.

Initial Design Point:

|  | M1,2 | M3,4 | M5,6 |
| --- | --- | --- | --- |
| W | 16.72um | 320nm | 40um |
| L | 430nm | 2.57um | 2.43um |
| ID | 20uA | 20uA | 40uA |
| gm/ID | 16 | 20 | 9.2 |
| VDsat | 101.4mV | 62.6mV | 169.5mV |
| Vov | 25.91mV | -22.39mV | 170.8mV |
| Vstar | 125mV | 100mV | 217.4mV |

Simulation and Design Iteration:

Figure 13 5T OTA Schematic After Iterations

After very Few iterations these are the values, I reached for all transistors that satisfy all specs.

Final Design Point:

|  | M1,2 | M3,4 | M5,6 |
| --- | --- | --- | --- |
| W | 20um | 3.5um | 40um |
| L | 450nm | 330nm | 2.2um |
| ID | 20uA | 20uA | 40uA |
| gm/ID | 11 | 5 | 5 |
| VDsat | 156.1mV | 303.9mV | 300mV |
| Vov | 131.5mV | 307.3mV | 389.2mV |
| Vstar | 181.8mV | 389.5mV | 422mV |

These values were calculated at different VDS and VSB values matching the assumptions we did during the design stage but they differ greatly when doing the simulation as we’ll see in the following results.

Simulation and Results:

Figure 14 Testbench Schematic for first part

OP Analysis:

Figure 15 OP Analysis Results for all transistors

Figure 16 DC Voltages Annotated in Testbench

Figure 17 Voltages Annotated on 5T OTA Schematic

• Is the current (and gm) in the input pair exactly equal?

Yes, the current is exactly equal in both transistors of the input pair.

• What is DC voltage at VOUT? Why?

Vout=0.75003

Vout=VDD-VGS3,4=1.8-1.05=0.75V

There’s a virtual short-circuit between the drain and gate of M3 cause both Vout and V1 to have the same voltage.

Differential Small Signal:

Figure 18 Diff Gain (dB )vs Frequency

Figure 19 Diff Gain Values

Hand Analysis:

Using Values from OP Analysis:

Avd=gm1,2ro1,2 //ro3,4=54.46→34.7213 dB

|  | Simulation | Hand Analysis |
| --- | --- | --- |
| Gain | 53.415 | 54.46 |
| Gain (dB) | 34.55 | 34.7213 |

Hand analysis and simulation results agree with each other and satisfy the required spec.

CM Small Signal:

Figure 20 CM Gain (dB) vs Frequency

Figure 21 CM Gain Values

Hand Analysis:

Using Values from OP Analysis:

AvCM=12gm3,4RSS=12gm3,4ro5=5.0578×10-3→-45.92 dB

|  | Simulation | Hand Analysis |
| --- | --- | --- |
| Gain | 4.095e-3 | 5.0578e-3 |
| Gain (dB) | -47.75 | -45.92 |

Hand analysis and simulation results agree with each other.

CMRR:

Figure 22 CMRR vs Freq from simulation

Hand Analysis:

Using Diff and CM gain values from last 2 parts:

CMRR=AvddB-AvCMdB=80.6413 dB

|  | Simulation | Hand Analysis |
| --- | --- | --- |
| Gain (dB) | 82.31 | 80.6413 |

Hand analysis and simulation results agree with each other and satisfy the required spec.

Diff Large Signal:

Figure 23 VOUT vs Vid

What is the value of Vout at VID = 0? Why?

Vout@Vid=0=0.75,  Same Value as Vout calculated During OP Analysis, This is the Quiescent point of the amplifier

Figure 24 Derivates of Vout vs Vid

Figure 25 Value at the Peak

The Value of the derivative of Vout vs Vid is the DC Gain, the value at the peak=58.87 is observed to be slightly higher than Avd=53.415 but still comparable to it

CM Large Signal:

Figure 26 CMIR From Simulation

VCMLOW=0.8609V,   VCMHIGH=1.516V

The result meets the required spec

Figure 27 GBW vs VCIM wiht VCIM Annotated

The result for GBW meets the required specifications inside the input range!

Part 3 Conclusion:

| Specification | Required | Achieved |
| --- | --- | --- |
| Supply voltage | 1.8V | 1.8V |
| Load | 5pF | 5pF |
| Open loop DC voltage gain | >= 34dB | 34.55 |
| CMRR @ DC | >= 74dB | 82.31 |
| Phase margin | >= 70o | 89.7o |
| CM input range – low | <= 1V | 0.86V |
| CM input range – high | >= 1.5V | 1.516 |
| GBW | >= 10MHz | 10.413MHz |

Part 4: Closed Loop OTA Simulation:

Figure 28 Closed Loop Testbench

Figure 29 OP Point in Feedback Configuration

• Is the current (and gm) in the input pair exactly equal? Why?

No, there is a slight mismatch between them.

• Calculate the mismatch in 𝐼𝐷 and gm.

IDmis=ΔID=0.325μA

gmmis=Δgm=4.04μs

Loop Gain:

Figure 30 STB Simulation Testbench

Figure 31 STB Simulation Results

Figure 32 Loop Gain (dB) vs Frequency

Figure 33 Loop Gain Phase vs Frequency

Loop Gain=35.34dB

Compare DC gain and GBW with those obtained from open-loop simulation

From Simulation:

β=Loop GainAOL=0.913

Figure 34 Closed Loop Gain in dB vs Frequency

The Amplifier work as a buffer in this configuration thus the gain is approximately equal to 0dB (Unity Gain).

Figure 35 Closed Loop Bandwidth

GBWCL=BWCL≈10.348MHz

Compared to open loop simulation the gain her is unity but we can se