Lab 7

OTA Design

Part 1: gm/ID Design Charts

NMOS Charts:

Figure 1 gm/gds vs gm/id

Figure 2 ID/W vs gm/ID

Figure 3 gm/Cgg vs gm/ID

Figure 4 VGS vs gm/ID

PMOS Charts:

Figure 5 gm/gds vs gm/ID

Figure 6 ID/W vs gm/ID

Figure 7 gm/Cgg vs gm/ID

Figure 8 VGS vs gm/ID

Part 2: OTA Design

Required Specifications:

| Technology | 0.18um CMOS |
| --- | --- |
| Supply voltage | 1.8V |
| Load | 5pF |
| Open loop DC voltage gain | >= 34dB |
| CMRR @ DC1 | >= 74dB |
| Phase margin | >= 70o |
| CM input range – low | <= 1V |
| CM input range – high | >= 1.5V |
| GBW | >= 10MHz |

Design Observations:

- The Gain required isn’t too large and easily achievable using a single stage OTA

- The CMIR required is close to the Supply Rail, Thus an NMOS input stage is preferred.

- The specification can be met using a 5T-OTA topology

We have to size 3 pairs of MOSFETS.

Figure 9 5T OTA Topology

Input Transistors:

GBW≈GMCL=gm1,22πCL=10MHz→gm1,2=314.16μs≈320μs

AOL≥34dB→50.11≈gm1,2ro2Assuming Both NMOS and PMOS have the same gds

→ro=312.5KΩ

AssumegmID=15 In Monderate Conversion→ID=21.3μA≈20μA

Assuming the Voltage Drop is divided equally across all 3 transistors in branch, VDS of each transistor = 0.6V

M1,2 Experience Body Effect, thus VSB = 0.6

We may assume a lesser voltage drop at the bottom transistor to 0.3 thus Vds = 0.9V

Using these three parameters we can get the width and length of the input pair quickly using ADT:

Figure 10 Intial Values for the input pair from SA

L=430nm     ,   W=16.72μm

These are acceptable values for the Input Pair and decent for initial design!

PMOS Current Mirror Load:

Using CMIR-Max:

CMIRMAX=1.5=VDD+Vth1,2-Vgs3,4 Using Value of Vth,2 from SA

→ Vgs3,4=1.09≈1.1V

Using assumptions from the previous part:

ro=312.5KΩ    ,    VDS=0.6V    ,    ID=20μA

Plugging in these Values in ADT SA we get:

Figure 11 Initial Values for the PMOS Current Mirror Load Pair from SA

L=320nm     ,   W=2.57μm

These are acceptable values for the Input Pair and decent for initial design!

Tail Current Mirror Sizing:

For the purposes of this design, I will use a simple current mirror at the tail.

Using the spec for CMRR:

CMRR=74dB=Ad-ACM→ACM=-40dB

→ACM=0.01≈12gm3,4ro5→1ro5=gds5≈2μs (Using Values of gm3,4 from SA)

Using the Spec for CMIR-Low:

CMIRLow=1V=Vgs1,2+V5*→V5*=100mV Extremely Low

ID=40μs→From previous assumption, twice the current flows in the tail source

Plugging in these Values into ADT SA:

Figure 12 Initial Values for the Tail Current Mirror Load Pair from SA

L=2.43μm     ,   W=674.7μm

The Value for the width is extremely high especially considering I need to multiply it by 4 for the mirroring ratio.

Thus I will assume an initial Value of W of 40um and slightly increase the Width of M1,2 then iterate the design till I reach acceptable Values for the transistor that satisfy the specifications.

Initial Design Point:

|  | M1,2 | M3,4 | M5,6 |
| --- | --- | --- | --- |
| W | 16.72um | 320nm | 40um |
| L | 430nm | 2.57um | 2.43um |
| ID | 20uA | 20uA | 40uA |
| gm/ID | 16 | 20 | 9.2 |
| VDsat | 101.4mV | 62.6mV | 169.5mV |
| Vov | 25.91mV | -22.39mV | 170.8mV |
| Vstar | 125mV | 100mV | 217.4mV |

Simulation and Design Iteration:

Figure 13 5T OTA Schematic After Iterations

After very Few iterations these are the values, I reached for all transistors that satisfy all specs.